9” Straw wattle

Description

Strawwattledirect.com Straw wattles are a HDPE photodegradable netting filled with an organic fill media consisting of agricultural straw.  Each wattle will be 25’ in length and secures with hog ring staples or tied at each end.  Wattle netting will consist of HDPE netting with a diamond shaped aperture measuring .5 inches x .5 inches, weighing .5 ounces per foot when fully expanded. Fill material will be 100% chopped agricultural straw weighing approximately 2 lbs. per foot.

Physical Specifications

Diameter	9 inches (22.86cm)

Length		25.00 feet (7.62 meters)

Weight 		50 pounds (22kilograms)

Installation

Wattles will be installed on contours with 24” stakes at a maximum 5 foot on centers (6 stakes per wattle) to insure intimate contact with the existing ground.  Where required wattles will be trenched 2-3 inches into existing grade to insure intimate contact with soil and eliminate undermining.